Storyline:

- long-term research in Kaunertal

- intensive paraglacial adjustment - one of the fastest retreat rates (Gepatschferner) in AT

- monitored retreat and paraglacial adjustment for 5 years now primarily for science communication

- CamDo setup - low cost GoPro Sensor

- direct response to retreat

- seasonality of para glacial adjustment

FIELD SETUP

- Which Sensors?

- Which housing?

- How many/which GCPs?

- How many cameras?

- Ideal Camera Positions?

- Where to buy equipment?

PROCESSING

COOPERATION

- Additional ressources available?

- Additional personel available?

- How to go on from here?

High-end Custom Equipment

- https://www.backmarket.at/de-at/p/spiegelreflex-sony-alpha-a7-r-iv-nur-gehause-schwarz/bfda1040-0a70-494d-acec-da8abdcf1f43?utm_source=google&utm_medium=cpc&utm_campaign=AT_SA_SHOP_G_GEN_Others_RSC&gclid=EAIaIQobChMIxPKYtLfsjwMVExkGAB3B-wzwEAQYASABEgLtjfD_BwE&gad_source=1&gad_campaignid=21642533414&gbraid=0AAAAACfZYsroaf6ItQcHUjMs5VTZaiuPU&l=10

- https://www.backmarket.at/de-at/p/sony-objektiv-sel100400gmsyx/e101113e-7e12-4f8a-b1f5-d1ac055440d3?l=10#

- https://thesextonco.com/collections/ignite-single-camera-enclosures

- Quick summary

Use two (or more) rugged time-lapse cameras rigidly fixed to a stable mount, synchronized and geometrically calibrated, with surveyed GCPs/targets in the scene, robust power/housing for alpine conditions, and a processing pipeline that combines stereo photogrammetry (SfM / MVS) ± image correlation. Key geometry rules: maintain good overlap, a sensible stereo baseline (rule-of-thumb baseline:distance ≈ 1:30 for distant scenes), and keep the convergence angle modest (≈ 5–15°) to maximize depth precision. Geologische Publikationen+1

- What the system must do

- Capture consistent, high-quality images over months/years (manual exposure, locked focus, RAW preferred). ResearchGate

- Provide two (or more) views of the same scene from fixed (unchanging) geometry so you can do stereo reconstruction and dense 3-D time series. isprs-archives.copernicus.org

- Be robust to cold, wind, water, snow, condensation and wildlife/ theft. ScienceDirect

- Have power & data strategy (solar + battery, local storage or cellular uplink). enlaps.io

- Camera technology — what to choose

- Time-lapse / interval capability (independent of user interface). Look for true time-lapse controllers or integrated devices. (Enlaps, Brinno, CamDo for GoPro controllers are typical examples). enlaps.io+2brinno-mkt+2

- Sensor & resolution: larger sensor and higher resolution → better S/N and more pixels for disparity. Prefer APS-C or full-frame if budget allows; small sensors still usable for short distances. Geologische Publikationen

- Lens: prime (fixed focal length) over zoom; small aperture changes and sharper optics help photogrammetry. Record focal length and keep it constant (no auto-zoom).

- Manual control & RAW: manual exposure, fixed white balance, fixed focus (or repeatable autofocus), shoot RAW if possible to maximize dynamic range and post-processing. ResearchGate

- Durability: rated for low temperatures, or put inside heated enclosure. Look for IP66/67 housings, desiccant packs and lens hoods/wipers. ScienceDirect

- Synchronization: hardware trigger is ideal for exact simultaneity; alternatives: NTP/GPS time stamping + tightly coordinated intervals. Hardware sync reduces reprojection errors between the pair. isprs-archives.copernicus.org

Product classes to consider:

- All-in-one rugged timelapse units (solar, cellular, enclosure, cloud): Enlaps (Tikee), Brinno long-term TL cameras — easiest to deploy but may lack stereo hardware sync. Good for monoscopic monitoring or when only qualitative 2D is required. enlaps.io+1

- Action cameras + controllers: GoPros with CamDo controllers — cheap, robust, flexible, but image quality & lens correction need attention. CamDo adds solar and scheduling. CamDo Solutions

- Custom rigs (DSLR / mirrorless or industrial cameras): highest quality and control — use intervalometer, external triggers, weatherproof housings. This is the best choice for stereo photogrammetry if you need cm-level change detection. Geologische Publikationen

- Stereo industrial cameras / depth cameras (Oak-D, ZED, e-consystems stereo modules): useful for short-range depth sensing and quick prototyping, but most depth cams are designed for tens of meters and may not give photogrammetry accuracy at 100s of meters. Evaluate case-by-case. Luxonis+1

- Geometry & mounting (positioning, baseline, angles)

This determines achievable depth resolution — spend time here. Principles:

- Stable rigid mount: mount both cameras to the same rigid bar/tripod/pole so relative pose doesn’t change (use anti-vibration mounts). Anchor the mount to bedrock or deep foundation if possible. isprs-archives.copernicus.org

- Baseline (distance between cameras): rule-of-thumbs:

  - A classic “1:30” baseline:distance rule is often used for stereo scenes (baseline ≈ distance / 30). For example, if your target is ~150 m away, baseline ≈ 5 m. Longer baseline increases depth precision but reduces overlap and near-range coverage. Use the ratio appropriate to your target distance and desired depth precision. shortcourses.com+1

  - Convergent (toe-in) vs parallel: modest convergence angles (≈ 5–15°) are often recommended to improve depth geometry without creating strong distortion. Keep bisector elevation angle reasonable to avoid extreme foreshortening. Tandfonline

- Field of view & overlap: aim for ≥70–80% overlap in the region of interest so photogrammetric matching is robust.

- Elevation & sightlines: place cameras high enough to avoid being buried by snow or occluded by seasonal vegetation or construction (choose locations out of avalanche runout unless secured). Use anti-snow shields and consider sun angles (avoid direct sun into the lens). ResearchGate

How to pick baseline/angles in practice:

- Compute expected depth error using stereo geometry: Z=B⋅fdZ = \dfrac{B \cdot f}{d}Z=dB⋅f​ (f = focal length in same units as baseline, d = disparity in pixels). Depth precision improves with larger baseline B and focal length f and decreases with distance Z and disparity noise. Use focal length (mm) + sensor pixel pitch to compute expected disparity and then expected depth uncertainty. Guidance on the math and tradeoffs is in stereo vision docs. ni.com+1

- Calibration & GCPs (!!!)

- Intrinsic calibration (lens focal length, principal point, distortion) — do this in the lab with checkerboards or calibration software (one time but re-check seasonally or after any mechanical shock). Medium

- Extrinsic calibration (relative pose between cameras) — either fix rigidly and measure in the lab, or compute via bundle adjustment during processing. Use a known-geometry calibration target if possible. isprs-archives.copernicus.org

- Ground Control Points (GCPs): absolutely necessary for georeferencing and absolute accuracy. Place coded targets or painted targets across the foreland (stable, visible in images all year). Survey GCPs with RTK-GNSS (cm-level) or total station. More GCPs → better absolute accuracy. Geologische Publikationen

- Scale bars & temporary targets: useful for internal scale checks; put scale bars in the scene if you expect big vertical changes.

- Repeatable markers: choose targets that remain visible through seasons (painted rocks, stakes with high-contrast targets).

- Data capture parameters & timing

- Interval: depends on process pace — for rapid paraglacial activity use 10–30 min; for seasonal changes daily may suffice. More frequent images = better DIC / motion tracking but more storage/power cost. Frontiers

- Shutter speed: fast enough to avoid motion blur (wind, snow). If light is low, raise ISO but watch noise. Use consistent exposure (manual mode) to keep photogrammetry matching stable. ResearchGate

- Time stamping: embed precise timestamps (GPS or NTP) in filenames/metadata. If you need exact simultaneity for stereo, use hardware trigger or ensure both cameras capture within <1 s (preferably <100 ms). isprs-archives.copernicus.org

- File format & compression: RAW or high-quality JPEG. RAW gives best post processing but increases storage. Keep original files; store checksums.

- Redundancy: if possible, run overlapping cameras (3rd camera) to allow multi-view reconstructions and to guard against single camera failure. ScienceDirect

# Power, communications & housing

- Power: solar panels + Li battery sized for site latitude/season (winter insolation low) + heater for battery in extreme cold. Consider over-spec’ing battery and panel for alpine winter. ScienceDirect+1

- Enclosure: IP66/IP67, UV-resistant polycarbonate windows, heated to avoid condensation, desiccant inside. Add a lens hood and wiper if heavy snow/rain expected. ScienceDirect

- Communications: cellular 4G if uplink needed (Enlaps does this), or local storage with periodic field downloads. Uplink simplifies monitoring but watch SIM/data costs and coverage. enlaps.io

- Security: bolt the mount and enclosure; use locks, camouflaging, or remote alarms if theft is a concern.

- Processing pipeline & software

- Primary approaches:

  - Stereo photogrammetry / multi-view SfM + MVS (Agisoft Metashape, Pix4D, OpenMVG + OpenMVS, COLMAP) to get dense point clouds and DEMs. Geologische Publikationen

  - Image correlation / DIC on monoscopic sequences for velocity fields (PyTrx, DIC plugins). Works well for flow velocity time series. Frontiers

  - Deep learning / stereo networks are emerging for low-cost 4D monitoring (see recent low-cost stereo + deep learning work). isprs-archives.copernicus.org

- Registration through time: use GCPs and stable bedrock patches to align successive reconstructions.

- Uncertainty estimation: always compute point cloud/DEM comparisons with independent checkpoints to estimate vertical/horizontal accuracy (report MAD/ RMSE). USGS guidance is practical here. Geologische Publikationen

- Expected accuracy & precision

- Order-of-magnitude: with careful stereo geometry, good calibration, and RTK GCPs, cm to decimeter vertical accuracy is achievable for short-to-moderate distances (tens of metres). For targets at >>100 m accuracy often degrades to decimeter–meter scale unless you use very long baseline / telephoto optics and very precise disparity measurement. The depth precision depends strongly on baseline, focal length and distance (see stereo geometry). Geologische Publikationen+1

- Practical note: If you need sub-10 cm vertical sensitivity across a 100–300 m foreland you’ll need: good baseline (several meters), high focal length (telephoto), high resolution sensor, many well-distributed GCPs and careful calibration. Otherwise expect larger uncertainty.

- Field deployment checklist

- Pick camera bodies (DSLR/mirrorless or industrial) + identical lenses.

- Design rigid stereo bar and mount; choose baseline with 1:30 rule as starting point and adjust for distance. shortcourses.com

- Ensure hardware sync (trigger) or highly accurate time stamps. isprs-archives.copernicus.org

- Survey and install ≥4 GCPs visible in both cameras; mark with coded targets. Survey with RTK GNSS. Geologische Publikationen

- Weatherproof enclosures, solar + battery sized for worst-case, heater/insulation, desiccant. ScienceDirect

- Pre-calibrate cameras in the lab; record intrinsics.

- Field test for several days: check image overlap, exposure stability, synchronization, and visibility of GCPs.

- Implement data transfer or schedule downloads; verify checksums.

- Run initial processing (SfM) and compute checkpoint errors; tweak baseline/angles if needed. Geologische Publikationen

- Manufacturer / product pointers

- All-in-one timelapse + cloud: Enlaps (Tikee) — good turnkey option for long deployments and cellular upload. enlaps.io

- Economical timelapse: Brinno (TLC series) — cheap, long-term recording but limited photogrammetry control. brinno-mkt

- GoPro + controller: CamDo — proven for time-lapse and solar for action cams. Good for low-cost stereo rigs of GoPros. CamDo Solutions

- Custom / research rigs: Use mirrorless/DSLR with intervalometer + industrial enclosures; consult the open-source replicable stereo system papers for designs. isprs-archives.copernicus.org+1

- Stereo modules / depth camera makers (if short range): Luxonis (Oak-D), e-consystems stereo modules — useful for short-range automated depth maps and prototype testing. Luxonis+1

- Useful literature & resources

- USGS: Camera system considerations for geomorphic applications of SfM photogrammetry — practical field guidance. Geologische Publikationen

- Ioli et al. / ISPRS & replicable open-source stereo systems for glacier monitoring (low-cost stereo designs). isprs-archives.copernicus.org

- Blanch et al., Fixed photogrammetric systems for natural hazard monitoring (assembly & code examples). NHESS

- PyTrx and other open-source tools for monoscopic velocity extraction. Frontiers

- Common pitfalls

- Losing sync between cameras (causes systematic stereo errors). isprs-archives.copernicus.org

- Not enough GCPs or targets that get obscured seasonally. Geologische Publikationen

- Battery/solar undersizing for winter months. ScienceDirect

- General idea – current research direction

- Today, automated rockfall detection from webcams or time-lapse cameras combines computer vision, machine learning (especially deep learning), and photogrammetric workflows.

- The three main approaches used in research and practice are:

- Time-lapse / fixed-camera photogrammetry

- → Used for change detection and volume quantification (2D, 3D, or even 4D).

- Computer vision + deep learning

- → CNNs or object detection models (YOLOv5–YOLOv8, DINO, Mask R-CNN, etc.) trained to detect moving rocks or rockfall events automatically.

- Multi-sensor systems

- → Cameras combined with radar, seismic, or LiDAR sensors for redundancy and all-weather capability.

- Why deep learning and photogrammetry dominate now

- Deep-learning models have recently become accurate enough to detect small, fast-moving objects like falling rocks.

- Time-lapse photogrammetry provides spatially explicit measurements — enabling 3D mapping and volume estimation of rockfall sources.

- Combining both methods gives robust automatic detection and precise geolocation.

- Typical “state-of-the-art” pipeline

Step 1. Data acquisition

- Fixed RGB webcam or time-lapse camera (optionally thermal).

- Stable mounting, fixed viewpoint, calibrated lens.

- Capture frequency depending on application (e.g., 1–10 Hz for video, 1–60 min for time-lapse).

- Optional co-located sensors (radar or seismic) as triggers.

Step 2. Pre-processing

- Image alignment/registration (temperature drift, wind movement).

- Radiometric correction (lighting normalization).

- Filtering of night or foggy images.

Step 3. Event pre-filter / motion detection

- Frame differencing, background subtraction (MOG2, KNN, etc.) or optical flow to identify candidate motion zones.

- Reduces computational cost before running deep models.

Step 4. Object detection / classification

- Apply trained deep-learning detectors (e.g., YOLOv8, DINO, Mask R-CNN).

- Models are often retrained on site-specific image datasets to recognize rocks and debris movements.

Step 5. Tracking

- Use multi-object trackers (SORT, DeepSORT, Kalman filters) to track trajectories, estimate velocity, impact zones, etc.

Step 6. False-positive filtering

- Apply heuristics or classifiers based on:

- object size

- speed / direction

- persistence in consecutive frames

- optional cross-check with radar or seismic triggers.

Step 7. Post-processing (georeferencing / 3D analysis)

- With stereo or time-lapse photogrammetry: compute 3D displacement fields and rockfall volumes.

- Software used: OpenMVG/OpenMVS, Agisoft Metashape, MicMac, etc.

Step 8. Alerting / visualization

- Set thresholds (e.g., ≥ N detections per minute).

- Send alarms or logs to dashboards, databases, or GIS systems.

- Common issues & solutions

- Shadows, lighting changes, moving vegetation --> Background modeling, temporal filters, data augmentation

- Small, fast objects --> Train multi-scale detectors, use higher frame rates

- Nighttime or fog --> Add radar or thermal imaging

- Lack of training data --> Transfer learning, synthetic datasets, active learning

- Processing delay --> se lightweight models (YOLOv8n, MobileNet) on Jetson or similar edge devices

- Tools and libraries

- Preprocessing & motion detection --> OpenCV

- Deep learning --> PyTorch, TensorFlow

- Object labeling --> CVAT, LabelImg

- Photogrammetry --> OpenMVG, OpenMVS, Agisoft Metashape

- Tracking --> SORT, DeepSORT

- Visualization & alerting --> Python (Dash, Flask), Grafana, QGIS integration

- OpenCV – Computer Vision and Preprocessing Framework

OpenCV is a C++/Python library optimized for real-time image and video analysis.
In rockfall detection systems, OpenCV is typically responsible for the front-end of the pipeline: preparing, cleaning, and identifying regions of interest (ROIs) in imagery.

| Task | Scientific Function | Purpose |
| --- | --- | --- |
| Image registration | Detect and correct sub-pixel shifts between frames using feature matching (e.g. ORB, SIFT, ECC alignment). | Ensures camera shake, temperature drift, or mount vibration do not create false motion signals. |
| Illumination normalization | Histogram equalization, CLAHE, or gamma correction. | Reduces effects of changing light, shadows, and cloud cover. |
| Motion detection / change detection | Frame differencing, Background Subtraction (MOG2, KNN), or Optical Flow (Farneback, Lucas-Kanade). | Identifies moving pixels or zones likely corresponding to rockfall events. |
| Masking static areas | Static masks based on terrain outline or sky region. | Reduces false positives from non-relevant regions. |
| ROI extraction | Crops bounding boxes or patches with detected motion. | Feeds relevant sub-images to YOLOv8 for classification. |
| Noise reduction | Morphological operations, Gaussian blur. | Removes isolated noise points (snow, insects, vegetation motion). |

- YOLOv8 – Deep Learning Object Detection and Classification

YOLOv8 (by Ultralytics, 2023) is a state-of-the-art real-time object detection model based on the “You Only Look Once” (YOLO) architecture family. It combines convolutional neural networks (CNNs) and anchor-free detection heads to predict object locations and classes in a single inference pass.

| Step | Description |
| --- | --- |
| Input | An image or video frame (e.g., 640×640 pixels). |
| Feature extraction (Backbone) | A CNN (e.g., CSPDarknet or ConvNeXt variant) encodes visual features (edges, textures, object parts) at multiple scales. |
| Neck (Feature pyramid) | Combines multi-scale features so small and large objects (e.g., small falling rocks vs. large blocks) are both detected. |
| Detection head | Predicts bounding boxes, class labels (e.g., “rock”, “dust”, “debris”), and confidence scores directly from feature maps. |
| Loss optimization | Uses a combination of localization loss (e.g. CIoU), classification loss (cross-entropy), and objectness loss. |
| Output | For each detected object: bounding box coordinates (x, y, w, h), class label, and confidence value. |

https://chatgpt.com/s/t_69022f86cbcc8191bb0f1910d05ae3c4

- Technical Report: High-Resolution Rockfall Monitoring in Alpine Environments Using Time-Lapse Photogrammetry

- 1.0 Introduction: A Paradigm Shift in Geohazard Monitoring

The continued expansion of development and population into mountainous regions globally necessitates a strategic and robust approach to geohazard management. Monitoring the stability of rock slopes is of paramount importance for assessing hazards and mitigating risks to infrastructure, transportation corridors, and human life. For decades, this task has been dominated by high-end technologies like terrestrial lidar scanning, which provide exceptional accuracy. However, these traditional methods present significant limitations, including high infrastructure and equipment costs, logistical difficulties with deployment in remote or inaccessible locations, and the impracticality of conducting massive, periodic deployments required for effective long-term monitoring.

This report introduces terrestrial time-lapse photogrammetry as a modern, cost-effective, and powerful alternative that addresses many of these challenges. By leveraging advances in digital photography, computer vision, and autonomous systems, fixed photogrammetric stations can deliver high-resolution, high-frequency data on slope dynamics. The purpose of this document is to provide a comprehensive technical overview of the methodology, equipment, and operational considerations for establishing such a system. Drawing upon recent research and field-based case studies, this report details the complete workflow, from system architecture and field deployment to data processing and performance analysis, demonstrating that photogrammetry is not merely a low-cost substitute but a viable and, in some cases, superior monitoring strategy. The discussion will begin with the foundational principles that enable this powerful transformation of two-dimensional images into dynamic three-dimensional landscape models.

- 2.0 Foundational Principles: From 2D Images to 4D Landscape Analysis

An effective monitoring system is built not just on capable hardware, but on a clear understanding of the principles that govern data acquisition and interpretation. For a time-lapse photogrammetry system, a grasp of the underlying computer vision and geoscientific concepts is essential for optimizing system design, ensuring data quality, and extracting meaningful insights. This section covers the core concepts that transform a sequence of simple photographs into precise, four-dimensional models of landscape change over time, where the fourth dimension is the temporal evolution of the monitored surface.

At the heart of this technology are the Structure-from-Motion and Multi-View Stereo (SfM–MVS) algorithms. These revolutionary computer vision techniques are the primary enablers for creating detailed 3D models from a series of overlapping 2D images. The SfM algorithm first identifies common feature points across multiple images to simultaneously solve for the 3D structure of the scene and the camera positions and orientations from which the images were taken. Following this, the MVS algorithm uses this geometric information to generate a dense, high-resolution point cloud that represents the surface topography with remarkable detail. This process eliminates the need for direct distance measurements, allowing a flexible and scalable approach to 3D reconstruction.

Complementing this technological foundation is the science of Geomorphometry, defined as the quantitative analysis of the land surface. Geomorphometry provides the theoretical framework for interpreting the digital topography generated by SfM–MVS. It offers a suite of analytical tools to translate raw elevation data into geomorphologically significant features and processes. By quantifying surface parameters such as slope, aspect, curvature, and roughness, geomorphometry allows us to identify and measure phenomena like erosion, deposition, and deformation with objective, repeatable methods. This quantitative approach moves analysis beyond simple visual inspection, enabling objective and repeatable measurements of landscape evolution. These foundational principles directly inform the practical design and construction of the physical monitoring hardware, which is detailed in the following section.

- 3.0 System Architecture: Components of a Fixed Photogrammetric Station

This section serves as a practical guide to the design and assembly of an autonomous photogrammetric monitoring station. The primary objective is to create a robust, field-ready system capable of sustained, long-term operation in the harsh and often remote environments characteristic of alpine regions. A successful system must be resilient, energy-efficient, and capable of automated data acquisition and transfer. To this end, the system architecture is broken down into four essential hardware subsystems: imaging, control, power, and housing.

- 3.1 Imaging Subsystem: Selecting the Right Sensor

The selection of the camera and lens is the most critical decision in system design, as it directly determines the potential accuracy and spatial resolution of the final 3D models. The choice represents a trade-off between performance, cost, and complexity.

| System Type | Key Attributes |
| --- | --- |
| High-End Systems | Sensor Resolution: Very high (e.g., 30 Mpx with Canon EOS 5D Mark IV or Nikon DSLRs).<br>Accuracy Potential: Capable of achieving change detection thresholds of a few centimeters, comparable to lidar.<br>Cost: Significantly higher initial investment. |
| Low-Cost Systems | Sensor Resolution: Lower (e.g., 8 Mpx with Raspberry Pi Camera Module V2).<br>Accuracy Potential: Suitable for many applications, but with a change detection threshold 3 to 4 times higher than high-end systems when deployed at similar distances.<br>Cost: Very low, enabling wider deployment. |

Lens selection is equally important. In the Peru case study, 85 mm fixed-zoom lenses were used to achieve an optimal balance between field of view and ground sampling distance at the target cliff face. Manual focus is typically employed and set during installation to ensure consistency across all captured images.

- 3.2 Control and Processing Unit: The System's Brain

The central control unit orchestrates all system functions, enabling autonomous operation. In the referenced field applications, a single-board computer—specifically, the Raspberry Pi—serves this role. Its primary responsibilities include:

- Automating the image capture schedule.

- Managing the transfer of image files from the camera's memory to its own storage.

- Adding precise timestamps to filenames for chronological organization.

- Executing scripts to transmit the captured images to a remote server for processing and archiving.

- 3.3 Power Management Subsystem: Ensuring Autonomous Operation

To operate independently in remote locations without access to the electrical grid, the station requires a self-sustaining power system. This is typically achieved with a solar-based setup comprising several key components:

- Power Source: A Solar Panel (e.g., 10 W) to convert sunlight into electrical energy.

- Energy Storage: A 12 V Battery to store energy, ensuring the system remains operational during nighttime and periods of low sunlight.

- Charge Regulation: A Solar Charge Controller to manage the charging process, protecting the battery from overcharging and deep discharge.

- Voltage Regulation: DC-DC Converters to step down the 12 V supply from the battery to the specific voltages required by the system components, such as 7.5V for a DSLR camera and 5V for the Raspberry Pi.

- Power Switching: Programmable Relays (e.g., Witty Pi) that act as intelligent switches, turning power-intensive components like the camera on only when needed for image capture and off immediately afterward to conserve energy.

- 3.4 Housing and Ancillary Equipment

Protecting the sensitive electronic components from harsh environmental conditions is critical for long-term operational reliability. Waterproof housing is essential for both the camera and the control unit box. Field experience has also highlighted the importance of several ancillary components:

- A lens hood is a simple but effective addition to mitigate issues like water drops accumulating on the lens filter and to reduce lens flare from direct sunlight, both of which can render images unusable.

- When using metal boxes to house the control unit, an external antenna is necessary. The metal enclosure acts as a Faraday cage, which blocks Wi-Fi and cellular signals, and an external antenna is required to ensure reliable data telemetry.

The careful selection and integration of these hardware components are the first step, but their successful application hinges on strategic deployment in the field.

- 4.0 Field Deployment and Operational Considerations

Successful monitoring is contingent not only on well-designed hardware but also on strategic field deployment and diligent, ongoing maintenance. Real-world applications invariably present a range of practical challenges that must be anticipated and managed to ensure data quality and system longevity. The following considerations, synthesized from field experience, are critical to operational success.

- Site Selection and System Geometry: The physical placement of the cameras is fundamental. Sites must be accessible for installation and maintenance while providing a stable, unobstructed view of the area of interest. A convergent image network, where cameras are positioned to view the target from multiple angles, is crucial for robust 3D reconstruction and minimizing occlusions.

- Environmental Limitations: The quality and availability of data are heavily influenced by environmental conditions. Image acquisition is inherently limited to daylight hours. Furthermore, adverse weather such as heavy rain or dense fog can obscure the target, while direct sunlight can cause lens flare, rendering images unusable for photogrammetric processing.

- Data Telemetry: The remote nature of many monitoring sites poses significant challenges for data transfer. In one case study, unreliable cellular signal quality necessitated a switch from an automated wireless transfer system to manual data collection on a monthly basis. This highlights the need for robust on-site data storage as a backup.

- Focus Management: Maintaining sharp focus on the target rock slope is a critical and sometimes problematic operational detail. Field examples have shown cameras incorrectly focusing on foreground vegetation instead of the distant cliff face. Technical issues, such as a faulty connector in a time-lapse system, have also been identified as a cause of focus loss, underscoring the need for periodic checks.

- System Maintenance: Autonomous systems are not entirely self-sufficient and require periodic maintenance to remain operational over long periods. In one documented case, rust was observed on a control unit due to water infiltration, requiring intervention. Experience shows that systems deployed for over a year consistently require attention to ensure their continued function.

Overcoming these field challenges allows for the consistent acquisition of raw data, which must then be processed through a rigorous computational pipeline.

- 5.0 Data Processing: From Raw Images to Change Detection

The conversion of raw time-lapse imagery into quantifiable 3D models of landscape change requires a rigorous, systematic, and repeatable data processing workflow. This computational pipeline is as critical to the monitoring effort as the field hardware itself, as it ensures that detected changes are real and not artifacts of inconsistent processing. This section outlines the key stages of this workflow, from automated data handling to the precise quantification of rock slope activity.

- 5.1 Automated Acquisition and Transfer Workflow

To manage the high volume of data generated by a time-lapse system, an automated code pipeline is essential. The workflow developed by Blanch (2023) exemplifies such a system, orchestrating the journey of an image from capture to final storage:

- Image Capture: The control unit triggers the camera according to a predefined schedule.

- Image Transfer: The captured image is immediately transferred from the camera's memory card to the control unit (e.g., Raspberry Pi).

- Timestamping: The file is renamed with a precise timestamp to ensure chronological integrity.

- Temporary Storage: The image is held in a temporary folder on the control unit.

- Upload to Server: The system attempts to upload the image to a remote server for secure, off-site storage and processing.

- Transfer to Local Backup: Upon successful upload, the image is moved to a local backup folder on the control unit as a secondary safeguard.

- 5.2 The MEMI Workflow for High-Precision 3D Models

To ensure that point clouds from different time periods are geometrically consistent, the Multi-Epoch and Multi-Imagery (MEMI) workflow is employed. This approach consists of two key strategies:

- Multi-Imagery: This involves capturing and using multiple photos from each camera during a single acquisition epoch (e.g., three photos taken a few minutes apart). The slight variations in lighting conditions between these photos allow the SfM algorithm to identify more feature points, resulting in a denser and more precise point cloud.

- Multi-Epoch: This technique involves including photos from two or more different time periods (epochs) in the initial image alignment step. By processing a stable reference epoch alongside the current epoch, this method ensures that the resulting point clouds share a consistent geometric framework and georeferencing.

Achieving this consistent geometry is the absolute prerequisite for reliable change detection, as it minimizes the risk of attributing geometric inconsistencies between epochs to spurious ground movement.

- 5.3 Quantifying Change

Once high-precision, co-registered point clouds have been generated for different dates, the final step is to quantify the changes between them.

- Comparison: The primary algorithm used for this task is the M3C2 (Multiscale Model-to-Model Cloud Comparison). M3C2 accurately measures the 3D distance between two point clouds along the direction of the local surface normal, providing a robust measure of change while accounting for surface roughness and orientation.

- Validation: The output of M3C2 is a cloud of change points. To distinguish significant geological events from processing noise, a minimum volume threshold is applied (e.g., 5 m³). Clusters of change points exceeding this threshold are then flagged for manual validation, a critical step where an analyst inspects the source photographs from the compared dates to confirm that a real rockfall or deformation has occurred.

- Volume Calculation: For validated rockfall events, volumes are calculated using an iterative Alpha-shape procedure. This algorithm is applied to each cluster of change points to create a watertight, manifold surface mesh, from which a precise volume can be computed.

The successful application of this processing workflow yields quantifiable data on slope dynamics, the performance and accuracy of which are analyzed in the next section.

- 6.0 Performance Analysis: Accuracy and Detection Capabilities

The ultimate measure of a monitoring system's value lies in its performance: its ability to produce accurate, reliable data and to detect and characterize meaningful geomorphic change. Analysis of the high-resolution photogrammetric systems described herein reveals a level of effectiveness that positions them as a viable alternative to more traditional, high-cost technologies.

These systems demonstrate remarkable accuracy. In the Puigcercós cliff case study, the methodology achieved a theoretical change detection threshold of 3.6 cm (0.036 m) across the entire monitored escarpment. In the central region of the cliff, where the imaging geometry and point density were optimal, this performance improved to an even more precise 1.4 cm (0.014 m). This precision is directly comparable to that of high-end survey instruments. At the same site, a previous study using terrestrial lidar reported a standard deviation of 0.0168 m between scans (Abellán et al., 2010). The ability of the photogrammetric system to achieve a standard deviation of 0.007 m in its optimal viewing region (equating to a 0.014 m change detection threshold) demonstrates that high-end photogrammetry can deliver comparable accuracy at a significantly lower cost.

Beyond accuracy metrics, the systems have proven their capability in real-world applications to detect both subtle, pre-failure movements and overt rockfall events.

- Pre-failure Deformation: At the Puigcercós cliff, the system successfully identified a rock toppling process. By comparing point clouds over a 308-day period, the system detected and quantified pre-failure deformation, with movement values reaching up to 0.16 m at the top of the mobilized blocks. This capability is critical for early-warning applications.

- Rockfall Inventory Generation: The high frequency of data acquisition allows the system to build comprehensive inventories of rockfall events, detailing their individual size, shape, and time of occurrence. This data enables the creation of Magnitude–Cumulative Frequency (MCF) curves, which are essential for quantitative hazard assessment and for understanding the relationship between rockfall frequency and volume.

While the system's performance is impressive, the choice of data acquisition frequency introduces a critical dilemma regarding what processes are revealed and what may be obscured.

- 7.0 The High-Frequency Data Dilemma: Process Insight vs. Total Volume

The decision to monitor at a high temporal frequency, such as daily, introduces a critical trade-off between process insight and the comprehensive measurement of total volumetric change, particularly when spatial resolution is a limiting factor. The central question is: what are the benefits and drawbacks of high-frequency data when the system may not be able to resolve very small changes? Analysis of long-term monitoring campaigns reveals a fundamental dilemma that directly impacts the interpretation of results.

The primary advantage of high temporal resolution is the ability to identify and correctly interpret progressive, spatially correlated rockfalls. Field data from the Peru study site revealed that some large failures did not occur as single, instantaneous events. Instead, they evolved progressively over several months through a series of smaller, adjacent detachments. With lower-frequency monitoring (e.g., biannual or annual surveys), this entire sequence would have been misinterpreted as a single, large-volume event, obscuring the true mechanics of the slope's failure process. High-frequency data provides unparalleled insight into the evolution and behavior of an active slope.

The primary disadvantage, however, emerges when this high-frequency workflow is combined with a minimum detection threshold. If the system's spatial resolution is insufficient to reliably detect small changes, a daily comparison with a volume threshold (e.g., 5 m³) will fail to capture the cumulative effect of many smaller, sub-threshold events like gradual raveling. This can lead to a significant underestimation of total material loss. A striking finding from the Peru study illustrates this point perfectly: the total rockfall volume identified using a daily monitoring frequency was less than one-fourth of the total volume identified in a single, start-to-end comparison spanning 399 days.

Ultimately, the choice of monitoring frequency must be dictated by the primary monitoring objective. For understanding process mechanics, characterizing progressive failures, and generating true Magnitude–Cumulative Frequency relationships, high-frequency analysis is indispensable. Conversely, for estimating the total long-term erosion budget, especially in environments dominated by gradual raveling, lower-frequency start-to-end comparisons are superior for capturing total volumetric loss. This understanding paves the way for emerging technologies that promise to further enhance our monitoring capabilities.

- 8.0 Future Outlook: Integrating Artificial Intelligence and Advanced Methods

The field of geomorphological monitoring is in a period of rapid evolution, driven by the integration of artificial intelligence (AI) and other advanced computational techniques. These emerging methods promise to enhance the capabilities of existing monitoring systems, extract more information from image data, and overcome some of the current limitations in topographic representation.

An exciting development is a new AI-based workflow for tracking surface movement using monoscopic (single camera) time-lapse imagery, which avoids the need for repeated 3D model generation. This process involves two key steps:

- First, an AI model such as PIPs++ is used to track a grid of points through a 2D time-lapse image sequence. This model is robust to changes in appearance caused by lighting variations or even partial snow cover, allowing it to maintain a consistent track over time.

- Second, a novel method called GIRAFFE translates these 2D image trajectories into 3D object space. It achieves this by generating a synthetic, photorealistic image from a reference 3D point cloud (e.g., from an initial UAV survey) and then matching features between the real camera image and the synthetic one. This matching allows the precise camera pose to be determined, enabling the projection of the 2D tracked points into 3D coordinates.

The final output of this workflow is a set of 3D velocity vectors for numerous points across the landform's surface, providing a dense field of motion data from a single, fixed camera. While this AI-driven approach significantly enhances our ability to extract dynamic information from existing 2D and 2.5D data formats, the next fundamental evolution lies in improving the underlying topographic representation itself.

Looking further ahead, there is a recognized need to move beyond current 2.5D representations of topography, like Digital Elevation Models (DEMs), towards "true" 3D or volumetric models. A DEM is computationally efficient but fundamentally limited, as it can only assign a single height value to any given (x, y) location. This makes it impossible to accurately represent complex, real-world topography such as overhangs, arches, or intricate rock faces. Developing algorithms and data structures that can handle true 3D surfaces will be a critical step in advancing the fidelity of geomorphological analysis. These advancements, coupled with the proven methodologies discussed in this report, inform a set of final conclusions and recommendations.

- 9.0 Conclusion and Recommendations

This report has detailed the methodology, architecture, and performance of fixed-base, time-lapse photogrammetry for geohazard monitoring. The findings from recent research and field applications demonstrate conclusively that this approach is a highly effective, accurate, and cost-efficient method for achieving high-resolution, high-frequency monitoring of rock slopes. It offers a powerful tool for researchers, engineers, and hazard managers seeking to understand and mitigate risks in mountainous environments.

The core conclusions of this analysis can be summarized as follows:

- Viable Lidar Alternative: High-end photogrammetric systems can achieve change detection accuracies comparable to terrestrial lidar, when properly deployed and processed using advanced workflows. This performance, delivered at a significantly lower cost, effectively democratizes access to high-resolution 4D monitoring.

- Temporal Resolution is Key: The primary advantage of time-lapse photogrammetry is its ability to capture data at high temporal frequencies. This is crucial for correctly interpreting complex slope behaviors such as progressive failure, a dynamic that is often aliased or entirely missed by less frequent survey campaigns.

- Awareness of Limitations: The choice of monitoring frequency creates a critical trade-off. The effectiveness of high-frequency monitoring for capturing total volumetric change is highly dependent on the system's spatial resolution. Users must be acutely aware that when a minimum detection threshold is applied, the cumulative impact of gradual, low-magnitude events (e.g., raveling) can be significantly underestimated.

Based on these findings, a final recommendation is offered for optimizing monitoring strategies, particularly for systems that are limited in spatial resolution. A multi-temporal change detection workflow should be instituted. This approach would involve conducting high-frequency (e.g., daily) comparisons to capture rapid dynamics and understand process evolution, in conjunction with longer-term, rolling-window comparisons (e.g., weekly, monthly). This hybrid strategy would allow practitioners to capture both the nuanced, progressive behavior of the rock slope and the cumulative, long-term volume loss, thereby providing the most comprehensive assessment of the prevailing hazard.